# Oozham (ഊഴം)

"Your Turn" — a heavy-vehicle warning system for hairpin bends.

Built by PASU Labs for TinkerHub LAYOVER 2026 (TinkerSpace Calicut) — Theme: Travel

# 1. Problem Statement

Hairpin bends on ghat roads like Thamarassery Churam are physically narrow. A loaded truck or bus needs the full width of the road to navigate the curve. When a heavy vehicle swings into the wrong lane to make the turn and another vehicle enters from the opposite side, neither can easily reverse — the result is a deadlock that can last hours.

### Real-World Impact: July 2026, Thamarassery Ghat

A single multi-axle truck stuck at the 4th hairpin caused a 10+ hour gridlock

Thousands of travellers stranded; ambulances delayed

Airport passengers walked uphill to find other transport

Churam Samrakshana Samithi: jams could be avoided if heavy vehicles were stopped at both ends whenever one is stuck on the bend

Kerala recorded about 49,889 road accidents in 2025; experts point to road design, curves and enforcement gaps.

### Why Existing Systems Fail

| System | What it does | What it cannot do |
| --- | --- | --- |
| Valparai Ghat Sensors (Tamil Nadu, 5th hairpin) | Warns drivers a vehicle is coming | Does not decide who goes first |
| #RoadsThatHonk (Patnitop, J&K) | Audio/visual alert at bends | Does not decide who goes first |
| Oozham | Decides whose turn it is | — |

Existing systems convert a blind spot into a warning. Oozham converts a warning into a decision.

# 2. Proposed Solution

Oozham puts an AI camera before a hairpin bend. When it sees a heavy vehicle (truck or bus) approaching, it sends a warning over LoRa radio to a sign on the other side of the bend, telling drivers a large vehicle is turning and they should wait. Light vehicles like cars and bikes get no warning — they can share the bend width and pass normally.

### How it works

A reCamera watches the road and detects vehicles with a YOLO11n model

Truck or bus detected → the camera lights up its warning LEDs and flags "heavy" on its API

An ESP32 reads the API and raises a digital signal

That signal triggers a LoRa transmitter, which sends the warning wirelessly across the bend

A LoRa receiver on the far side drives a warning LED — blinking when the road is clear, solid when a heavy vehicle is coming

### Why LoRa

The two ends of a hairpin bend have no wired connection and no reliable mobile signal. LoRa gives a simple, low-power, long-range wireless link that works on ghat roads.

# 3. Technical Implementation

### Hardware used

| Component | Role |
| --- | --- |
| Seeed reCamera 2002w (YOLO11n) | Detects vehicles and classifies truck/bus as HEAVY. Its 4 onboard white LEDs light up as the local warning |
| ESP32 | Hosts a private WiFi network, polls the camera API every 200 ms, drives a digital pin HIGH on heavy |
| Xiao nRF52840 + RYLR998 (LoRa TX) | Transmitter. Sends "H" over LoRa while the ESP32 pin is HIGH |
| Arduino Nano + RYLR998 (LoRa RX) | Receiver on the far side of the bend. Drives the warning LED |
| Warning LED (Nano D2) | Blinks when the road is clear, goes solid while a heavy vehicle signal is active |

### End-to-end signal chain

reCamera (YOLO11n)  --truck/bus-->  HEAVY + 4 white LEDs ON
        |  Node-RED API  GET /api/heavy  ->  true/false
        v
ESP32 (WiFi AP "HeavyCam_AP", camera at fixed IP 192.168.4.2)
  polls API every 200 ms  ->  D0 = HIGH on heavy, LOW on any error
        |
        v
Xiao nRF52840 + RYLR998  ->  LoRa "H" packets while pin is HIGH
        |        (address 0, network 10, 115200 baud)
        v
Arduino Nano + RYLR998  ->  D2 LED
  no signal = blink  |  heavy = solid HIGH

### Design decisions

Fixed IPs: the ESP32 creates its own WiFi AP and the camera always sits at 192.168.4.2, so the API address never changes — no router needed on-site

Fail-safe output: if WiFi or the HTTP call fails, the ESP32 pin goes LOW — a broken link never looks like a heavy vehicle

Hold timing: the camera keeps "heavy" true for ~3 s after the last detection so brief flickers between frames do not flap the signal

Tolerant receiver: the Nano accepts partial/corrupted LoRa packets — any real signal activity counts, because 115200 baud over software serial is not perfectly clean

LED semantics: blink = road clear, solid = heavy vehicle approaching; it returns to blinking ~0.7 s after the signal stops

### Software

Node-RED on the reCamera: YOLO11n detection -> heavy classifier -> LED trigger + API endpoints (/api/vehicle, /api/heavy, /api/vehicle/code, /api/vehicle/raw)

ESP32 sketch: SoftAP + HTTP GET every 200 ms -> digitalWrite(D0)

Xiao transmitter sketch: reads D2 input, streams LoRa "H" while HIGH

Nano receiver sketch: LoRa RX -> D2 output with blink/solid logic

All firmware and the Node-RED flow are in the repository: github.com/PASUMBA/Oozham

# 4. Impact

### Prevents the deadlock, not just warns about it

Existing bend-warning systems only tell drivers something is coming. Oozham goes further: a solid warning on the far side tells drivers to hold back while a heavy vehicle uses the full width of the bend — the exact behaviour the Churam Samrakshana Samithi asked for.

### Direct answer to a real, documented disaster

The July 2026 Thamarassery gridlock stranded thousands for 10+ hours — including ambulances — because one truck got stuck

The cause was not bad driving; it was the absence of any system that could hold traffic when a heavy vehicle needed the whole road

### Who benefits

Travellers: hours saved on one of Kerala's busiest ghat roads

Emergency services: bends stay passable instead of sealing shut for hours

Environment: less fuel burned by thousands of vehicles idling in jams

### Hackathon track fit (TinkerHub LAYOVER 2026)

| Track | Fit | Why |
| --- | --- | --- |
| 05 Travel hardware & IoT | Primary | reCamera + ESP32 + LoRa + LED — a real roadside device, built and working |
| 03 Navigation & mobility | Primary | Controls who moves first so traffic keeps flowing on ghat roads |
| 02 AI-powered travel experiences | Secondary | YOLO11n edge AI classifies vehicles as HEAVY/LIGHT |
| 01 Travel utilities & tools | Secondary | Saves travellers hours stuck at Thamarassery Churam |
| 07 Accessibility & sustainable travel | Secondary | Keeps emergency routes open, cuts fuel wasted idling |

### What comes next

A second camera/sensor on the opposite approach for two-way detection

Full STOP/GO signboards instead of a single warning LED

Ambulance priority and control-room alerts

Solar power for off-grid ghat locations

Oozham detects heavy vehicles approaching a hairpin bend and warns the other side over LoRa — replacing driver guesswork with a clear signal and preventing the deadlocks that paralysed Thamarassery Churam for 10+ hours.

Sources: Onmanorama & Times of India (24 Jul 2026, Thamarassery gridlock) · New Indian Express (Kerala accidents 2025) · The Hindu (Valparai sensors) · HP Lubricants (#RoadsThatHonk)